Here the designer used his own way to label the parts that he used. You will need to trace the parts placement and relabel them according to the schematic.

The above board has an error in the silk screen. The JFET at the lower right corner is shown with the wrong orientation, needs to rotated 180 degrees. This design also uses a single zener D1, so do as described above. You will also need to run two ground wires(-V and GND) from the board to the star ground.